STANDARD OPERATING PROCEDURE

Use this form to document the Health and Safety information associated with the procedure.

| Procedure Title |  | Biofilm Assay |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dept | ABE |  |  | Bldg/Rm | 4210 Sukup |  | Supervisor | Adina Howe |

Procedure Overview (brief description of the project)

To determine if a bacteria isolate is able to produce a biofilm.

Health and safety information for materials used (briefly describe the hazards associated with the materials and/or equipment OR document your hazard assessment in Section I)

Bacteria used in this assay are typically isolated from the environment (plant or soil samples) and are poorly characterized. Pathogenicity is a very low but non-zero risk. Observe at least BSL1 precautions.

Crystal violet can stain, handle with protective clothing.

Our bottle of Volu Sol Crystal Violet Stain contains up to 6% ethanol which is flammable. It also contains <1% C.I. Basic violet 3.

Hazard Control Measures

Please select which type of lab coat, eye protection, and hand protection will be used (Lab coat, eye and hand protection, and closed toe/heel shoes must be selected as required by Section D of the ISU Laboratory Safety Manual.)

|  | Latex gloves |  | Insulated gloves |  | Face shield |  | Respirator |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nitrile gloves |  | Safety glasses |  | Lab coat |  | Fume hood |
|  | Neoprene gloves |  | Vented goggles |  | Apron |  | Biosafety cabinet |
|  | Vinyl gloves |  | Splash goggles |  | Dust mask |  | Glove box |
|  | Fully enclosed shoes |  |  |  | Flame resistant lab coat |  |  |

Other Control Measures

Wash hands before leaving the lab

Methods (Include step by step instructions detailing the process or attach this document to an existing method.)

Based heavily on O’Toole 2011

Growing a biofilm

- Grow isolates in preferred broth. For the SynCom isolates, R2A works well.

- Measure OD600 of isolates and dilute to 0.1 with preferred broth.

- Dilute that dilution 1:100 into fresh media.

- Add 100 µL of diluted bacteria to each well. Have 4-8 replicates for each isolate.

- Place in an incubator at the preferred temperature. For the SynCom isolates, 29°C works well.

- The incubation time will vary by isolate. For the SynCom isolates, 48 hours is used.

Staining the biofilm

- After incubation, fold several layers of paper towels together to the size of the plate and invert the plate, absorbing the cells and media in the paper towels. Shake and tap the plate to remove as much media as possible. Dispose of paper towel in biohazardous waste.

- Submerge the plate in a small tub of ddH2O water. Shake out the water. Repeat. This reduces background staining.

- Add 125 µL of a 0.1% solution of crystal violet in water to each well of the plate. (To make this solution from the bottle we have in the lab, add 1 mL from out bottle to 19 mL of water). A multi-channel pipette can be used.

- Incubate at room temperature for 10-15 min.

- Rinse 3-4 times as described in step 2. Shake out and blot vigorously on a stack of paper towels.

- Turn plate upside down and dry for a few hours/overnight (I’ve done over the weekend).

Quantifying the biofilm

- Add 125 µL of 30% acetic acid in water to each well of the plate to solubilize the CV.

- Incubate at room temperature for 10-15 min.

- Transfer 125 µL of the solubilized CV to a new plate.

- Read in plate reader at 550 nm using 30% acetic acid in water as the blank.

Waste Disposal Procedures

All bacteria waste should be sterilized before disposal.

First Aid Procedures

INHALATION: Remove to fresh air. If not breathing, give artificial respiration. If breathing

is difficult, give oxygen. Consult a physician

INGESTION: If swallowed, vomiting may occur spontaneously, but DO NOT INDUCE. If

vomiting occurs, keep head below hips to prevent aspiration into lungs. Never give

anything by mouth to an unconscious person. Take victim immediately to hospital. Rinse

mouth with water. Consult a physician

SKIN: Immediately flush skin with plenty of soap and water. Remove contaminated

clothing and shoes, wash before reuse. Consult a physician

EYE: Immediately flush eyes with plenty of water for at least 15 minutes. Continue rinsing

eyes during transport to hospital. Consult a physician

Spill/Release Containment, Decontamination, and Clean Up Procedures

Follow laboratory spill procedures.

Using Substances Requiring Special Procedures?    No    Yes

(If Yes; identify authorized personnel, designate a use area and specify specialized safety precautions here. Refer to Section B in the ISU Laboratory Safety Manual for details.)

| Written By | Lorien Radmer |  | Date | 1/15/26 |
| --- | --- | --- | --- | --- |
| Approved By |  |  | Date |  |

(PI or Lab Supervisor)

- HAZARD ASSESSMENT

Use the hierarchy of controls to document the hazards and the corresponding control measure(s) involved in each step of the procedure.

Consider elimination or substitution of hazards, if possible.

Engineering Control(s): items used to isolate the hazard from the user (i.e. fume hood, biosafety cabinet).

Administrative Control(s): policies/programs to limit the exposure to the hazard (i.e. authorizations, designated areas, time restrictions, training).

Required PPE:  indicate PPE including specific material requirements if applicable (i.e. flame resistant lab coat, type of respirator or cartridge).

| Task | Hazard | Engineering Control(s) | Administrative Control(s) | Required PPE |
| --- | --- | --- | --- | --- |

- TRAINING RECORD

Use the following table to record the training associated with this Standard Operating Procedure.

| Print Name | Signature | Trained By | Date |
| --- | --- | --- | --- |